5.13题

A．

B．

这条关键路径决定的CPE下界为3

C.

对于整数数据的关键路径决定的CPE的下界为1

D.

在关键路径上只涉及到浮点数的加法

5.15题

void unroll6(vec_ptr u,vec_ptr v,data_t *dest) { long i; long length=vec_length(u); long limit=length-5; data_t *udata=get_vec_start(u); data_t *vdata=get_vec_start(v); data_t sum0 = (data_t)0; data_t sum1 = (data_t)0; data_t sum2 = (data_t)0; data_t sum3 = (data_t)0; data_t sum4 = (data_t)0; data_t sum5 = (data_t)0; for (i=0;i<limit;i+=6) { sum0+=udata[i]*vdata[i]; sum1+=udata[i+1]*vdata[i+1]; sum2+=udata[i+2]*vdata[i+2]; sum3+=udata[i+3]*vdata[i+3]; sum4+=udata[i+4]*vdata[i+4]; sum5+=udata[i+5]*vdata[i+5]; } for (;i<length;i++) { sum0+=udata[i]*vdata[i]; } *dest=sum0+sum1+sum2+sum3+sum4+sum5; }

5.17题

（此题参考了标准答案后基本理解。）

word表示以八字节进行覆盖的变量。首先用单个字节进行覆盖直至对齐，然后用八字节的word进行覆盖。最后剩余的空间以单个字节进行覆盖到结束。

void *my_memset(void *s,int c,size_t n) { unsigned long word=0; unsigned char *schar; unsigned long *slong; size_t cnt=0; long i; if (n<3*sizeof(word)) { return basic_memset(s,c,n); } for (i=0;i<sizeof(word);i++) { word=(word<<8)¦(c&0xff); } schar=(unsigned char *)s; while ((unsigned long) schar % sizeof(word)!=0) { *schar++=(unsigned char)c; cnt++; } slong=(unsigned long *)schar; while (cnt<n-2*sizeof(word)+1) { *slong=word; *(slong+1)=word; cnt+=2*sizeof(word); slong+=2; } schar=(unsigned char *)slong; while(cnt<n) { *schar++=(unsigned char)c; cnt++; } return s; }

5.19题

void psum(float a[],float p[],long n) { long i; int v=0; for (i=0;i<n-1;i+=2) { int v1=a[i]; int v2=v1+a[i+1]; p[i]=v+v1; p[i+1]=v+v2; v+=v2; } for (;i<n;i++) { v+=a[i]; p[i]=v; } }